Ques 19)

| analogWrite() | analogRead() |
| --- | --- |
| Produces a PWM output (simulates analog output). | Reads an analog voltage from a sensor. |
| Used on PWM pins (e.g., 3, 5, 6, 9, 10, 11 on Arduino Uno). | Used on analog input pins (A0–A5 on Arduino Uno). |
| Accepts values from 0–255. | Returns values from 0–1023. |

### What is PWM?

PWM (Pulse Width Modulation) is a technique that rapidly switches a digital signal ON and OFF. By changing the duty cycle (the percentage of time the signal stays ON), it creates the effect of different output voltages.

Why is it used?

- To control LED brightness

- To control DC motor speed

- To control the power delivered to devices efficiently

### Practical IoT Examples

- analogWrite() example: In a smart street lighting system, an Arduino uses PWM to automatically adjust LED brightness at night, saving energy.

- analogRead() example: In a smart agriculture system, an Arduino reads a soil moisture sensor using analogRead() and turns on a water pump when the soil becomes dry.