### Sensor Calibration

Sensor calibration is the process of comparing a sensor's output with known reference values and adjusting the readings so they accurately represent the real-world measurement.

In IoT systems, calibration is important because sensors often have manufacturing variations, environmental effects, and aging that can cause inaccurate readings.

### Why is Calibration Important?

- Improves accuracy of sensor readings.

- Ensures reliable decision-making by IoT devices.

- Reduces measurement errors caused by sensor tolerances.

- Provides consistent readings between different sensors.

- Increases the reliability of automation systems.

For example, an uncalibrated soil moisture sensor may show 60% moisture even when the soil is actually much drier, causing incorrect irrigation.

## Calibrating an Analog Soil Moisture Sensor

An analog soil moisture sensor outputs an analog value (typically 0–1023 on an Arduino). These raw values vary from sensor to sensor, so calibration is needed.

### Step 1: Measure the Dry Value

- Keep the sensor completely out of the soil (air).

- Read the analog value several times.

- Record the average value.

Example:

- Dry soil (or air) = 850

### Step 2: Measure the Wet Value

- Insert the sensor into fully wet soil (or a glass of water if recommended by the manufacturer).

- Record the analog value.

Example:

- Wet soil = 350

### Step 3: Convert Raw Values to Percentage

## Two-Point Calibration Method

The two-point calibration method uses two known reference points to accurately map sensor readings.

### Point 1 (Low Reference)

- Measure the sensor in a known dry condition.

- Example:

  - Dry reading = 850

  - Moisture = 0%

### Point 2 (High Reference)

- Measure the sensor in a known wet condition.

- Example:

  - Wet reading = 350

  - Moisture = 100%

The sensor readings between these two points are then converted linearly into percentages.

| Condition | Raw Reading | Moisture (%) |
| --- | --- | --- |
| Dry Soil | 850 | 0% |
| Half Moist | 600 | 50% |
| Wet Soil | 350 | 100% |

## Advantages of Two-Point Calibration

- Simple and easy to perform.

- Improves measurement accuracy.

- Compensates for differences between individual sensors.

- Provides meaningful moisture values (0–100%) instead of raw ADC readings.

- Widely used in IoT-based smart irrigation and precision agriculture.